HỆ THỐNG MÃ GIẢ (PSEUDOCODE) CÁC THUẬT TOÁN ĐIỀU HÀNH
A-SMGCS KẾT HỢP FOLLOW-THE-GREEN (FTG)

Tài liệu kỹ thuật thuật toán phục vụ Báo cáo Nghiên cứu & Đồ án tốt nghiệp ngành Hàng không / CNTT

1. TỔNG QUAN KIẾN TRÚC THUẬT TOÁN TRONG HỆ THỐNG

Trong hệ thống điều hành và dẫn dắt tàu bay mặt đất A-SMGCS (Advanced Surface Movement Guidance and Control System) kết hợp công nghệ đèn thông minh Follow-the-Green (FTG), hệ thống không chỉ vận hành bởi một thuật toán tìm đường đơn lẻ mà là sự phối hợp của 3 module thuật toán cốt lõi:

Thuật toán 1 — Định tuyến tối ưu (Routing Engine): Thuật toán Dijkstra mở rộng có ràng buộc động (Constrained Dynamic Dijkstra Taxiway Routing). Xác định lộ trình lăn ngắn nhất trên đồ thị sân bay G = (V, E) theo chiều dài mét thực tế, tự động loại bỏ các nhánh đường lăn bị cấm/đóng, đường một chiều hoặc đoạn đường đang xuất hiện sự cố ngoại vật (FOD).

Thuật toán 2 — Dẫn dắt & Điều khiển đèn (Guidance & Control): Thuật toán Điều khiển phân đoạn đèn Follow-the-Green và Thanh dừng Stop Bar (Dynamic FTG Segment & Stop-Bar Control). Bật sáng chuỗi đèn xanh (Centerline Taxiway Lights) dẫn dắt cuốn chiếu trước mũi tàu theo phân đoạn (block), tự động ngắt đèn sau đuôi tàu bay để tiết kiệm năng lượng và tránh gây nhầm lẫn cho tàu bay sau, đồng thời tự động kích hoạt vạch dừng đỏ (Red Stop Bar) tại các điểm chờ đường băng hoặc trước vị trí sự cố.

Thuật toán 3 — Giám sát & Giải tỏa xung đột (Surveillance & Safety Assurance): Thuật toán Giám sát xung đột giao lộ và Duy trì phân cách an toàn (A-SMGCS Conflict Detection & Separation Assurance). Quét khoảng cách tọa độ Euclid thời gian thực giữa các tàu bay tại các nút giao cắt nguy hiểm, phân định quyền ưu tiên di chuyển (Right-of-Way Priority) và tự động chỉ định tàu bay dừng chờ trước vạch an toàn nhằm triệt tiêu hoàn toàn nguy cơ va chạm mặt đất.

2.1 Mã giả Constrained Dynamic Dijkstra Taxiway Routing

Algorithm 1: Constrained Dynamic Dijkstra Taxiway Routing ────────────────────────────────────────────────────────────────────── Input: Airport directed graph G = (V, E) with edge physical length w(e) = length(e) [meters] Origin node s ∈ V (Parking Stand / Runway Exit), Destination node d ∈ V (Holding Point / Stand) Set of impassable edges E_blocked ⊂ E (FOD incidents, closed taxiways, temporary maintenance) Output: Optimal taxiway route P = (v_1, v_2, ..., v_k) where v_1 = s, v_k = d (or null if unreachable) ────────────────────────────────────────────────────────────────────── 1: Initialization: 2: for all v ∈ V do 3: dist[v] ← ∞, prev[v] ← null 4: end for 5: dist[s] ← 0 6: PriorityQueue Q ← {(s, 0)} // Min-Priority Queue sorted by distance 7: VisitedSet S ← ∅ 8: while Q is not empty do 9: u ← Extract-Min(Q) 10: if u ∈ S then continue 11: S ← S ∪ {u} 12: if u = d then break // Target reached 13: // Traverse outbound valid edges compliant with operational constraints 14: for each edge e = (u, v) ∈ E outgoing from u do 15: if e ∈ E_blocked or status(e) ∈ {closed, restricted} then 16: continue // Skip blocked or restricted segments 17: end if 18: alt ← dist[u] + w(e) 19: if alt < dist[v] then 20: dist[v] ← alt 21: prev[v] ← u 22: Insert-Or-Decrease-Key(Q, (v, alt)) 23: end if 24: end for 25: end while 26: // Backtrack optimal trajectory 27: if dist[d] = ∞ then return null 28: P ← empty_list, curr ← d 29: while curr is not null do 30: Prepend curr to P 31: curr ← prev[curr] 32: end while 33: return P

2.2 Mã giả Dynamic Follow-the-Green (FTG) Lighting and Stop-Bar Control

Algorithm 2: Dynamic Follow-the-Green (FTG) Lighting and Stop-Bar Control ────────────────────────────────────────────────────────────────────── Input: Aircraft a with route edge sequence P_a = (e_1, e_2, ..., e_m) and current edge index idx_a Set of all airport taxiway edges E_all Set of dynamic incident / FOD blocked edges E_blocked ⊂ E_all Runway holding points and conflict stop-bar set H_stop ⊂ E_all Guidance lookahead window size N_lookahead (typically 2–3 blocks ahead) Output: Dynamic light status map L: E_all → {GREEN, RED, OFF} ────────────────────────────────────────────────────────────────────── 1: for each edge e ∈ E_all do 2: // Step 1: Safety Stop Bar Activation (Runway Protection & Incident Safety) 3: if e ∈ E_blocked or e ∈ H_stop then 4: L[e] ← RED // Illuminate red stop bar 5: continue 6: end if 7: // Step 2: Correlate edge with aircraft assigned route 8: k ← FindIndex(e, P_a) 9: if k = -1 then 10: L[e] ← OFF // Outside of route trajectory 11: continue 12: end if 13: // Step 3: Progressive Dynamic Segment Lighting (Follow-the-Green block guidance) 14: if k < idx_a then 15: L[e] ← OFF // Trailing segment deactivation behind aircraft tail 16: else if k ≥ idx_a and k ≤ idx_a + N_lookahead then 17: L[e] ← GREEN // Active guidance block ahead of aircraft nose 18: else 19: L[e] ← OFF // Distant segments kept dark for power & clarity 20: end if 21: end for 22: return L

2.3 Mã giả A-SMGCS Junction Conflict Detection & Separation Assurance

Algorithm 3: A-SMGCS Junction Conflict Detection & Separation Assurance ────────────────────────────────────────────────────────────────────── Input: Active fleet A = {a_1, a_2, ..., a_n} where each a_i has 2D coordinate Pos(a_i) = (x_i, y_i) Minimum safety separation margin D_sep [meters] (standard ICAO clearance zone) Deterministic Right-of-Way hierarchy function Priority(a_i) (runway exit > arrival > departure) Active runway occupancy map Occ_RWY: {NORTH, SOUTH} → {AircraftID ∪ null} Output: Updated tactical movement state State(a_i) ∈ {TAXIING, HOLDING}, GuidanceSpeed(a_i) [kts] ────────────────────────────────────────────────────────────────────── 1: for each active aircraft a_i ∈ A do 2: // Stage 1: Runway Incursion Prevention (Single-occupant invariant) 3: target_rwy ← GetRunwayCorridor(NextEdge(a_i)) 4: if target_rwy ≠ null and Occ_RWY[target_rwy] ∉ {null, ID(a_i)} then 5: State(a_i) ← HOLDING, GuidanceSpeed(a_i) ← 0 6: TriggerAlert(HOLD_AT_STOP_BAR, a_i, target_rwy) 7: continue 8: end if 9: // Stage 2: Junction Separation & Conflict Detection 10: conflict_found ← false 11: for each aircraft a_j ∈ A (j ≠ i) do 12: dist ← √((x_i - x_j)² + (y_i - y_j)²) 13: if dist < D_sep then 14: conflict_found ← true 15: // Priority Resolution: lower priority aircraft yields to higher priority 16: if Priority(a_i) < Priority(a_j) then 17: State(a_i) ← HOLDING, GuidanceSpeed(a_i) ← 0 18: TriggerAlert(YIELD_RIGHT_OF_WAY, a_i, a_j) 19: else 20: State(a_i) ← TAXIING // Higher priority proceeds unimpeded 21: end if 22: break 23: end if 24: end for 25: // Stage 3: Autonomous Conflict Deconfliction & Resume 26: if not conflict_found and State(a_i) = HOLDING and target_rwy = null then 27: State(a_i) ← TAXIING, GuidanceSpeed(a_i) ← EffectiveSpeed(a_i) 28: TriggerNotification(RESUME_TAXI, a_i) 29: end if 30: end for 31: return {State, GuidanceSpeed}

3. BẢNG TỔNG HỢP KÝ HIỆU & ĐỘ PHỨC TẠP TÍNH TOÁN

| Thuật toán | Chức năng trong A-SMGCS | Độ phức tạp | Tần suất thực thi |
| --- | --- | --- | --- |
| Algorithm 1 (Routing) | Định tuyến đường lăn tối ưu & né sự cố ngoại vật FOD | O(¦E¦ + ¦V¦ log ¦V¦) | Khi cấp huấn lệnh ban đầu hoặc khi có sự cố động |
| Algorithm 2 (Guidance) | Điều khiển phân đoạn đèn xanh FTG & Stop Bar đỏ | O(¦E_all¦) | Mỗi chu kỳ quét khung hình (Frame-by-frame 60Hz) |
| Algorithm 3 (Surveillance) | Quét phân cách giao lộ & chống xâm nhập đường băng | O(N²) với N là số tàu bay | Mỗi chu kỳ mô phỏng (Tick dt = 0.05s - 0.1s) |

Kết luận học thuật: Bộ ba thuật toán trên tạo thành một kiến trúc điều hành hoàn chỉnh theo tiêu chuẩn ICAO Doc 9830 và Eurocontrol về A-SMGCS Cấp độ 2+ (Routing, Guidance, Surveillance). Việc định dạng mã giả theo quy chuẩn quốc tế ở trên hoàn toàn tương thích để chèn trực tiếp vào chương Phương pháp nghiên cứu hoặc Thiết kế thuật toán của Khóa luận/Báo cáo tốt nghiệp.